PMRADIO V1.4

by

Paul Ratcliffe

(c) Nov. 1999

This program is designed for use with the Reveal RA300 FM Radio Card and

also both the old and the new versions of the RadioTrack card manufactured

by AIMS Labs.

The program is freeware and as such:

USE OF THIS PROGRAM IS ENTIRELY AT YOUR OWN RISK.

INSTALLATION

Unzip the archive contents into a temporary directory and run the supplied

INSTALL.CMD program. This requires REXX support to be installed. The

program will prompt you to select the language, target directory, type of

radio card and will also prompt for an I/O port if this is a first time

installation.

Alternatively, you may install manually by following the instructions below.

Copy the device driver FMRADIO.SYS and applications PMRADIO.EXE and

RADIO.EXE to a directory of your choice.

The OS/2 CONFIG.SYS file must be modified to load the device driver by

inserting a line like the following:

DEVICE=x:\path\FMRADIO.SYS [/P:port] [/V] [/RT]

where x:\path is the drive and directory in which you installed FMRADIO.SYS

port    is the radio card port address (in hex.).

The port addresses supported are 30F and 20F. The /P parameter is optional

and if not supplied, will default to address 30F.

The port address must match that which you have configured on the card

itself - jumper JP1 on the Reveal RA300.

The device driver is Resource Manager compliant and will not install if

some other device has already claimed the ports (the hardware actually

consumes 4 ports: 30C-30F or 20C-20F).

The /V parameter is optional. It may be specified if you don't wish the

device driver to turn the radio volume down when shutting down OS/2.

The /RT parameter is optional. It is required if and only if you have the

new version of the AIMS Labs. Radiotrack card. Users of the old version of

this card or the Reveal RA300 must not use this parameter.

You must reboot (in order to load the device driver) before you can use the

applications.

If you are upgrading from PM Radio 1.02 or earlier, you should run

INSTALL.CMD in order to update the PM Radio INI file.

OPERATION

PM Radio is, as its name implies, a Presentation Manager application. It

can be started from the Workplace Shell, an OS/2 text mode session or even

directly from a DOS session.

The program is designed to operate much like a normal car radio, with

up/down tuning, scanning and volume controls, and preset memories which can

be activated at the touch of a button.

It is possible to drive the program entirely from the keyboard if the user

prefers - it is not necessary to have a mouse or other pointing device.

There is support for drag and drop colour and font changes, and the program

will maintain this information for the various states of controls which use

more than one colour/font.

TUNING

The frequency can be altered in steps of 100kHz (coarse tuning) or 25kHz

(fine tuning) between 87.5 MHz and 108.0 MHz using the tuning buttons.

Alternatively, the Frequency menu item can be selected which presents a

dialog box where you can enter a frequency directly or use the spin

buttons. Right clicking on the frequency window itself also brings up the

dialog box.

The colour of the text in the frequency window changes colour to indicate

whether a station is tuned correctly or not. The default colours are red

indicating off tune, changing to cyan when tuned correctly.

VOLUME

The volume control has 16 steps as that is all that the hardware supports.

A Mute function is also available, as sound is still audible at the minimum

volume level. Changing the volume automatically unmutes the sound.

The Dim function reduces the volume by a user configurable number of volume

steps. It is possible to change the volume level without affecting the Dim

state. Left clicking on the Dim window undims the radio. Right clicking on

the Dim window brings up the dialog box for setting the Dim level. Right

clicking on the Mute button either dims or undims as appropriate.

MEMORIES

The radio has 20 memories which may be selected at a touch of a button. The

default colour for the button text is black, changing to blue when the

station programmed in to that button is tuned in. Unallocated memories are

grey.

Each memory contains the frequency and volume assigned to a station and

optionally a name.

To store, edit and clear memory information, the Memory menu item is

selected. This brings up the dialog using the currently selected memory.

Right clicking on a preset button also brings up the dialog using the

details for the button clicked on.

The Load Current button loads the details for the currently tuned station

into the selected memory.

Undo restores the settings for this memory to what they were before the

current editing operation.

Clear is used to erase a memory.

Multiple memories may be changed at once by changing the Memory field.

Pressing Save saves all changes - Cancel aborts all changes.

OPTIONS

There are several which should be fairly self-explanatory.

Turning Full controls off removes the tuning and volume controls just

leaving the memory buttons. Keyboard control is always available even when

the controls are not displayed.

Mute whilst Tuning removes the momentary pops and splats which can occur

whilst tuning the radio.

Mute whilst Scanning removes the receiver noise which occurs whilst

scanning across the band.

Confirm on exit displays a confirmation box before the program is closed.

This allows you to exit with the radio on or off or to cancel the operation.

Switch off on exit automatically turns the radio off when the program is

closed.

Dim level allows the user to configure the number of volume steps by which

the volume is reduced when the Dim command is executed.

Events active is a global enable for any events that have been configured.

EVENTS

Events can be set up at a specified time of day (and optionally on every

hour or half hour thereafter) to set the radio into the Muted, Dimmed or

Normal volume states.

This may be useful e.g. to leave the radio on in the background dimmed and

be brought up to full level on the hour for the news.

Up to 1440 different events can be configured and can be individually

enabled or disabled.

A warning dialog is displayed if the user adds new active events without

enabling the global events option.

COMMAND LINE OPTIONS

There are five options available:

PMRADIO nn	Starts the program and selects memory nn.

PMRADIO /m	Forces the program to start in the minimized state.

PMRADIO /s	Forces the program to start in the restored state.

PMRADIO /p	Resets the fonts and colours to their defaults.

PMRADIO /r<host>[:<port>]

This controls a remotely hosted radio card. <host> is an IP

address or host name, and <port> is the port number of the

device server.

REMOTE OPERATION

It is possible to install the radio card hardware in one machine and

control it from one or even many other machines. My DEVSERV package is

required in order to use this. Refer to the DEVSERV documentation for

important installation and configuration instructions.

On the machine in which the radio card is located:

FMRADIO.SYS must be installed

DEVSERV.EXE must be started

PMRADIO.EXE is not required although it may be of use.

On every other machine which is required to control the radio card:

PMRADIO.EXE must be installed

DEVCLI.DLL must be in the same directory or somewhere along the LIBPATH

FMRADIO.SYS is not required.

Of course, you must have a working TCP/IP network before attempting this.

You can run the server and the client in one machine for testing purposes,

but this is obviously not as efficient as running without DEVSERV.

MISCELLANEOUS

An INI file is created in the same directory as the PMRADIO.EXE file to

hold all the settings required by the program. Nothing is written to the

OS/2 INI files.

Sometimes, it is possible for the hardware volume setting to get slightly

out of step with the software setting. The key Ctrl-Alt-Shift-R can be used

to reset this - the volume is reduced to minimum and then muted.

RADIO.EXE is a very simple command line driven program which you can use to

tune the radio, turn it on and off, and change the volume.

HISTORY

Version 1.4  3-11-99

Fix for minor display corruption when using NPSWPS.

Version 1.35  31-8-99

Bug fix for cosmetic problem when changing menu font size.

Version 1.34  7-8-99

Minor internal tweaks.

Add programming documentation for FMRADIO.SYS .

Version 1.33  23-5-99

Add ESC key to perform Mute/Unmute function.

Right clicking on the Mute button now dims or undims.

Version 1.32 14-5-99

Corrected installation bug.

Version 1.31  13-5-99

Italian translation now available.

Internal changes for DEVSERV.

Version 1.3  3-5-99

Major internal rewrite to clean up the window hierarchy.

Presentation parameters now supported on all dialogs.

Added support for controlling remote radio cards across a TCP/IP network.

Spanish translation now available.

Version 1.2  6-6-98

Added an installation program.

Non-English language support is now via a DLL rather than a separate

executable.

Added Mute while scanning option.

Added Switch off when exiting option.

Rewrote the Events dialog to make it state based rather than action

based.

Fixed a trap D and some cosmetic bugs in the Events dialog.

Fix a bug where multiple Mute/Dim entries appeared on the system menu

when starting minimized and with the minimize to desktop option selected.

Added Activate/Deactivate Events to system menu when minimized to

desktop.

Clicking on the Dim window now undims and right clicking brings up the

Dim Level dialog.

Version 1.1  10-8-97

Modified to allow tuning down to 87.5 MHz.

Multiple events are now possible instead of just 1.

Extensive support for drag and drop colour and font changes.

Add command line option to clear stored colour and fonts and return to

the defaults.

Device driver modified so that muting of radio on shutdown is optional.

Included simple command line program RADIO.EXE .

Version 1.02  12-7-97

Fixed a cosmetic problem - when muting or unmuting, the Mute button was

being redrawn before the icon was changed, causing a flash.

Added the option to disable muting during tuning.

German translation now available.

Version 1.01  26-6-97

Fixed an intermittent trap D when selecting the Memory menu item.

Fixed a problem with the resources - on some systems, the text of strings

was truncated.

Extracted the remaining text strings from the code to the resources in

preparation for a German translation.

Version 1.0  19-5-97

Initial release.

CREDITS

Thanks to:

Dave Liquorice <allsorts@nexus.demon.co.uk> for Beta testing and

suggestions.

Petric Frank <P.Frank@stgl.sel.alcatel.de> for the initial German

translation.

Xavier Caballe <xavi@caballe.com> for the initial Spanish translation.

Marco Cassili <sandman@flashnet.it> for the Italian translation.

Armin Schwarz <ASchw@worldnet.att.net> for help with the new Radiotrack

support.

OTHER

The German and Spanish translations are currently incomplete. If anyone can

help with finishing these, please contact me at one of the addresses below.

This program was initially developed under Warp 3 (without Win-OS/2) and

now is developed under Warp 4 using VisualAge C++ 3.0 for OS/2.

If you use this program could you please let me know. Also if you have any

comments, criticisms, bug reports or anything that could be improved.

I will reply to all messages received.

Paul Ratcliffe:	Internet: pmradio@orac.clara.co.uk

Internet: paul.ratcliffe@bbc.co.uk